MARMARA UNIVERSITY

FACULTY OF ENGINEERING

PHYS 1104

PHYSICS LABORATORY II

—————————————————————————————————————————

Non-Ohmic Device In A Circuit

—————————————————————————————————————————

Section: 12

Group: 5

Instructure: Non-Ohmic Circuit                                              Date: 03/21/2024      8:59 am

|  | Department | Student ID Number | Name & Surname |
| --- | --- | --- | --- |
| 1 | Computer Engineering |  |  |
| 2 | Computer Engineering |  |  |
| 3 | Computer Engineering |  |  |

# 1. Table of Contents

1. Table of Contents	2

2. Purpose	3

3. Theory	3

4. Experimental Setup	3

4.1. List Of Materials	4

4.2. Procedure	4

5. Calculations and Results	5

Part 1	5

6. Discussion and Conclusion	8

7. References	9

8. Appendix	10

Experiment Sheet	10

# 2. Purpose

The study conducted to investigate the differences between a tungsten filament bulb and a resistor with ohmic characteristics helps us understand the behaviors of different components in electrical circuits. A tungsten filament bulb consists of a tungsten wire heated by an electric current, and as the temperature of this wire increases, its resistance also increases. Therefore, the resistance-current relationship of the bulb is not linear, and the bulb exhibits non-ohmic behavior. On the other hand, a resistor with ohmic characteristics shows resistance directly proportional to the electric current, meaning that as the current increases, the resistance also increases or decreases. These different behaviors are important for circuit analysis and controlling electrical loads. By comparing the behavior of both components, this study can help us understand how different components work in electrical circuits.

# 3. Theory

According to the Ohm’s law, the resistance of a resistor varies linearly as the voltage applied

on it increases. This is called as Ohmic behavior and ohmic devices create a straight line on

a current versus potential difference (voltage) plot. Since a bulb made with the tungsten

filament has a resistance dependence on the temperature, the plot of current versus voltage will be like a parabola for the bulb. Here, the reason of the nonlinear resistance change is arisen from the scattering mechanism in metals which hinders the electron transport. As a result, the resistance of the device increases. Due to the tungsten metal has a large    temperature coefficient, the resistance of the filament bulb increases as the filament heats up. If the voltage applied on the bulb increases, the temperature and the brightness of the bulb also increases. Below, the resistance formula is given related to the temperature. Using this formula, one can find the resistance of the bulb at corresponding temperature.

Figure 1: Filament Bulb Circuit

𝑅=𝑅𝑟𝑜𝑜𝑚[1+𝛼(𝑇−𝑇𝑟𝑜𝑜𝑚)] (1)

Here, the α is the temperature coefficient of the tungsten and T is the temperature in Kelvin for the maximum terminal voltage you apply. 𝑇𝑟𝑜𝑜𝑚 is 293 K. 𝑅𝑟𝑜𝑜𝑚 is the resistance of the bulb at the room temperature before starting the experiment. 𝑅 is the resistance at maximum terminal voltage.

# 4. Experimental Setup

## 4.1 Equipment List

1. Ceramic Wirewound Res. (10Ω - 11W) x1

2. Digital Multimeter

3. Filament Bulb x1 (3.8V – 0.3A)

4. DC Voltage Source (0 – 30 V range) x1

5. Connection cables

## 4.2 Procedure

1. Build the circuit shown in the Figure 1.

2. Connect a resistance in series to the bulb. This prevents the heating and burning of the bulb.

3. Measure the resistance of the bulb at the room temperature.

4. Calculate allowable maximum resistance value of the bulb using equation 1. Note: Temperature coefficient, and melting point for tungsten are 4.5𝑥10−3𝐾−1 and 3695K respectively.

5. Calculate the maximum voltage value of the power supply by taking into account the maximum current allowed to pass through the lamp.

6. Calculate the voltage step by dividing the maximum power supply voltage by 15.

7. Set the power supply voltage to zero.

8. Increase the terminal voltage by the calculated step value to maximum source voltage, then reduce the voltage by the same step value to zero volts.

9. For each terminal voltage value, measure and record the potential differences of the resistor and bulb and current passing through the circuit simultaneously using the digital multimeter.

10. Using data in the Table 1, plot an I-V graph for the bulb and resistor.

11. In order to see the nonlinear behavior of the tungsten filament bulb, wait little enough the bulb to heats up.

# 5. Calculations and Results

## 5.1 Tables

|  | Voltage |  |  | Current (A) |
| --- | --- | --- | --- | --- |
|  | Power Source (Vs) | Bulb (Vb) | Resistor (Vr) |  |
| 1 | 1.5 | 0.268 | 1.007 | 0.0964 |
| 2 | 2.0 | 0.543 | 1.192 | 0.114 |
| 3 | 2.5 | 0.763 | 1.373 | 0.133 |
| 4 | 3.0 | 1.072 | 1.580 | 0.151 |
| 5 | 3.5 | 1.355 | 1.765 | 0.169 |
| 6 | 4.0 | 1.643 | 1.933 | 0.186 |
| 7 | 4.5 | 1.56 | 1.88 | 0.21 |
| 8 | 5.0 | 1.87 | 2.07 | 0.23 |
| 9 | 5.5 | 2.18 | 2.24 | 0.25 |
| 10 | 6.0 | 2.50 | 2.40 | 0.27 |
| 11 | 6.5 | 2.88 | 2.56 | 0.28 |
| 12 | 7.0 | 3.15 | 2.72 | 0.30 |
| 13 | 7.5 | 3.53 | 2.85 | 0.31 |
| 14 | 8.0 | 3.84 | 3.1 | 0.33 |
| 15 | 8.5 | 4.11 | 3.1 | 0.34 |
| 16 | 8.0 | 3.83 | 2.99 | 0.32 |
| 17 | 7.5 | 3.51 | 2.85 | 0.31 |
| 18 | 7.0 | 3.17 | 2.7 | 0.29 |
| 19 | 6.5 | 2.83 | 2.55 | 0.28 |
| 20 | 6.0 | 2.51 | 2.70 | 0.26 |
| 21 | 5.5 | 2.83 | 2.55 | 0.24 |
| 22 | 5.0 | 2.50 | 2.39 | 0.23 |
| 23 | 4.5 | 2.16 | 2.22 | 0.21 |
| 24 | 4.0 | 1.84 | 2.05 | 0.19 |
| 25 | 3.5 | 1.54 | 1.86 | 0.18 |
| 26 | 3.0 | 1.24 | 1.62 | 0.16 |
| 27 | 2.5 | 0.55 | 1.47 | 0.14 |
| 28 | 2.0 | 0.66 | 1.27 | 0.12 |
| 29 | 1.5 | 0.35 | 1.06 | 0.10 |

# 6. Discussion and Conclusion

# 7. References

- https://canvas.instructure.com/files/246788285/download?download_frd=1

- https://canvas.instructure.com/files/245848688/download?download_frd=1

# 8. Appendix

## Experiment Sheet