Marmara University Faculty of Engineering

PHYS 1103 PHYSICS LABORATORY I

Measurements Errors and Uncertainty

Section: 2

Group: 1

Submitted to: Arş. Gör. Hüseyin YALTIRIK                           Due Date: 29.10.2024

|  | Dept ID | Student I | Name / Surname | Contribution |
| --- | --- | --- | --- | --- |
| 1 | BIOE |  |  | Conclusion, Calculations, Results, Theory |
| 2 | BIOE |  |  | Conclusion, Calculations, Results, Theory |
| 3 | BIOE |  |  | Conclusion, Calculations, Results, Theory, Upload |

Table of Contents

1.	PURPOSE	3

2.	THEORY	3

3.	EXPERIMENTAL SETUP	3

5.	CALCULATIONS AND RESULTS	3

6.	CONCLUSION	6

7.	REFERENCES	11

8.	APPENDIX	12

# PURPOSE

The purpose of this experiment is to learn how to use a caliper and a ruler correctly. It aims to understand the difference in measurements between the caliper and the ruler, identify errors, and analyze the results by comparing them.

# THEORY

Using a ruler and caliper with different precision, even when the same measurements are made by three different people, can lead to differences in results.

# EXPERIMENTAL SETUP

## A List of Materials

# Ruler (margins of error 0.05 cm)

# Caliper (margins of error 0.01 mm)

# Precision scale

# Rectangular prism

# Cylinder prism

# Cube

# CALCULATIONS AND RESULTS

- Rectangular Prism

Measurements of the Rectangular Prism Using Ruler and Caliper

We all took turns measure the rectangular prism by using a ruler and a caliper. The results are given below.

By given equations and measured data, we are going to calculate average of each edge and calculate volume of rectangular prism.

Ruler (cm); b±Δb; a±Δa; c±Δc. 1; 5.40±0.05; 3.40±0.05; 4.50±0.05. 2; 5.40±0.05; 3.50±0.05; 4.50±0.05. 3; 5.50±0.05; 3.50±0.05; 4.50±0.05. Average; 5.43±0.05; 3.43±0.05; 4.50±0.05. Table 1.1 Measuring dimension of the object

Caliper (mm); b±Δb; a±Δa; c±Δc. 1; 55.20±0.01; 35.10±0.01; 45.00±0.01. 2; 55.10±0.01; 35.10±0.01; 45.20±0.01. 3; 55.15±0.01; 35.00±0.01; 45.00±0.01. Average; 55.15±0.01; 35.06±0.01; 45.06±0.01. Table 1.2 Measuring dimension of the object

From this average data of the measurement we calculate volume of the object.

|  | By ruler | By Caliper |
| --- | --- | --- |
| V± ΔV (cm3) | 83.82 cm3 | 87.32 cm3 |

Table 1.3 Results of the calculation of the volume of the object from dimensional measurements.

To make a calculation for density, we weigh the object three times and calculated the average. We used the digital scale for this.

By digital balance (in unit of gram) m ± Δ m. 1; 122.05 g. 2; 122.04 g. 3; 122.06 g. Average; 122.05 g. Table 1.4 Measuring mass of the object

To calculate the density of the object, according to calculations we fill this table.

| Density (g/cm3) | By ruler | By Caliper |
| --- | --- | --- |
| By digital balance | 1.46 g/cm3 | 1.40 g/cm3 |

Table 1.5 Calculating density of object

- Cylinder

Measurements of the Cylinder Using Ruler and Caliper

Next, we are going to repeat the initial process for cylindrical project. We all took turns measure the cylinder by using a ruler and caliper. The results are given below and we calculated the average of the cylinder’s edge.

| Ruler (cm) | D ±Δ D | H ±ΔH |
| --- | --- | --- |
| 1 | 4.3±0.05 | 6.0±0.05 |
| 2 | 4.4±0.05 | 5.9±0.05 |
| 3 | 4.5±0.05 | 6.1±0.05 |
| Average | 4.4±0.05 | 6.0±0.05 |

Table 1.6 Measuring dimension of the object

| Caliper(mm) | D ±Δ D | H ±ΔH |
| --- | --- | --- |
| 1 | 45.10±0.01 | 60.30±0.01 |
| 2 | 45.10±0.01 | 61.00±0.01 |
| 3 | 45.00±0.01 | 59.08±0.01 |
| Average | 45.06±0.01 | 60.13±0.01 |

Table 1.7 Measuring dimension of the object

With this information we are going to calculate values specially wanted from us.

|  | By ruler | By caliper |
| --- | --- | --- |
| V± ΔV (cm3) | 91.23 cm3 | 95.89cm3 |

Table 1.8 Results of the calculation of the volume of the object

To calculate the density, we weigh the object and calculated the average weight.

|  | By digital balance (in unit of gram) m ± Δ m |
| --- | --- |
| 1 | 131.30 g |
| 2 | 131.32 g |
| 3 | 131.31 g |
| Average | 131.31 g |

Table 1.9 Measuring mass of the object

From this data we will calculate the density.

| Density (g/cm3) | By ruler | By caliper |
| --- | --- | --- |
| By digital balance | 1.44 g/cm3 | 1.37 g/cm3 |

Table 2.0 Calculating density of object

- Cube

Measurements of the Cylinder Using Ruler and Caliper

Lastly, we are going to repeat the previous process for cube project. We all took turns measure the cube by using a ruler and caliper. The results are given below and we calculated the average of the cube’s edge.

a

a

a

| Ruler (cm) | a ±Δ a |
| --- | --- |
| 1 | 4.5±0.05 |
| 2 | 4.5±0.05 |
| 3 | 4.5±0.05 |
| Average | 4.5±0.05 |

2.1 Measuring dimension of the object

| Caliper (mm) | a ±Δ a |
| --- | --- |
| 1 | 45.12±0.01 |
| 2 | 45.00±0.01 |
| 3 | 45.11±0.01 |
| Average | 45.11±0.01 |

2.2 Measuring dimension of the object

From this average data of the measurement we calculate volume of the object.

|  | By ruler | By caliper |
| --- | --- | --- |
| V± ΔV (cm3) | 91.13 cm3 | 91.55cm3 |

Table 2.3 Results of the calculation of the volume of the object

To calculate the density, we weigh the object and calculated the average weight.

|  | By digital balance (in unit of gram) m ± Δ m |
| --- | --- |
| 1 | 128.44 g |
| 2 | 128.46 g |
| 3 | 128.42 g |
| Average | 128.44 g |

Table 2.4 Measuring mass of the object

From this data we will calculate the density.

| Density (g/cm3) | By ruler | By caliper |
| --- | --- | --- |
| By digital balance | 1.41 g/cm3 | 1.40 g/cm3 |

Table 2.5 Calculating density of object

# CONCLUSION

In this experiment, we learned how to use calipers and rulers for measurements. As a result of this, we compared the differences between these two measurement tools.

First, we measured and weighed the objects using both the ruler and the caliper, then take notes and according to data we calculated their average measurements and grams. After that, we determined their volumes and calculated their densities. In addition, we compared of the differences between the results and saw the importance of the using the right measuring tool.

# REFERENCES

- https://www.youtube.com/watch?v=vkPlzmalvN4

- https://www.youtube.com/watch?v=0TLkSBD93ec

- https://www.youtube.com/watch?v=ySRN3yuZUT0

- https://www.youtube.com/watch?v=4hlNi0jdoeQ&t=9s

- https://www.youtube.com/watch?v=Tu-CzS3Uk-8

# APPENDIX