MARMARA UNIVERSITY

FACULTY OF ENGINEERING

COMPUTER ENGINEERING

PHYS1103

PHYSICS LABORATORY

EXPERIMENT 1

MEASUREMENTS, ERRORS AND GRAPHS

SXGXE1

| NAME | STUDENT NUMBER |
| --- | --- |

Submitted to: Hüseyin YALTIRIK

Experiment Date: 08.10.2019	Due Date: 15.10.2019

1-) PURPOSE

The purpose is to learn how to use caliper and micrometer; measure lenght, volume of a given object; determine depth of a given object; determine the possible errors in the measurements by ruler and caliper.

2-) THEORY

In Physical measurements, there are always some margin of error. We assume the ruler's margin of error is 0.05cm and the caliper's is 0.01mm. These values should be considered while making measurements.

Some informations about our calculations:

With ruler, margin of error (ΔX) is 0.05 centimeters.

With caliper, margin of error (ΔY) is 0.01 milimeters.

Area of a circular surface= A= π⋅r2 (r for "radius" and h for "height")

Volume of a cylinder: V= π⋅r2⋅h

Margin of error for volume calculation: ∆V= ±2⋅∆rr+∆hh

3-) EXPERIMENTAL SETUP

The materials that we used in this experiment are:

- Caliper

- Ruler

- A plastic cylinder

- Pen and paper

Before the start, we got the informations about how to use the materials in our experiment.

4-) CALCULATIONS & RESULTS

This is equation for calculating volume cylinder with margin of error.

V+∆V= π⋅r2⋅h±2⋅∆rr+∆hh

r = radius, h = height, ∆r = margin of error for radius, ∆h = margin of error for height

∆r and ∆h equals to 0.05 cm when using ruler.

∆r and ∆h equals to 0.01 mm when using caliper.

So if we put these values to equations we get two new equations.

For ruler

V+∆V= π⋅r2⋅h±(2⋅h+r20⋅h⋅r)

For caliper

V+∆V= π⋅r2⋅h±(2⋅h+r100⋅h⋅r)

Measurement results with caliper in centimeters

Table 1 Measurement results with ruler

| No | Radius (cm)±∆r | Height (cm)±∆h |
| --- | --- | --- |
| 1 | 2.965±0.05 | 9.43±0.05 |
| 2 | 2±0.05 | 5.66±0.05 |
| 3 | 2±0.05 | 0.4±0.05 |

Table 2 Measurement results with caliper

| No | Radius (mm)±∆r | Height (mm)±∆h |
| --- | --- | --- |
| 1 | 29.4±0.01 | 94.2±0.01 |
| 2 | 18.5±0.01 | 55.1±0.01 |
| 3 | 19.3±0.01 | 3.1±0.01 |

Let’s calculate the result with these values.

Table 3 Volume of cylinders (calculated with caliper)

| No | Radius (mm)±∆r | Height (mm)±∆h | Volume (mm3)±∆V |
| --- | --- | --- | --- |
| 1 | 29.4±0.01 | 94.2±0.01 | π ⋅ 81,422.712±0.000786429 |
| 2 | 18.5±0.01 | 55.1±0.01 | π ⋅ 18,857.975±0.00126257 |
| 3 | 19.3±0.01 | 3.1±0.01 | π ⋅1,154.719±0.00426208 |

Table 4 Volume of cylinders (calculated with ruler)

| No | Radius (cm)±∆r | Height (cm)±∆h | Volume (cm3)±∆V |
| --- | --- | --- | --- |
| 1 | 2.965±0.05 | 9.43±0.05 | π ⋅ 82.90125175±0.03902903975 |
| 2 | 2±0.05 | 5.66±0.05 | π ⋅ 22.64±0.05883392226 |
| 3 | 2±0.05 | 0.4±0.05 | π ⋅1.6 ±0.175 |

5-) CONCLUSION & DISCUSSION

In this experiment, we discovered that compared to a ruler, a caliper can be used for more precise measurements. Also, we learned to calculate margin of error for cylinder's volume.

6-) REFERENCES

https://www.varsitytutors.com/hotmath/hotmath_help/topics/volume-of-a-cylinder