MARMARA UNIVERSITY

FACULTY OF ENGINEERING

PHYS 1104

PHYSICS LABORATORY II

—————————————————————————————————————————

Capacitor Charge and Discharge

—————————————————————————————————————————

Section: 12

Group: 5

Instructure:   Capacitor Charge and Discharge                           Date: 04/04/2024      8:59 am

|  | Department | Student ID Number | Name & Surname |
| --- | --- | --- | --- |
| 1 | Computer Engineering |  |  |
| 2 | Computer Engineering |  |  |
| 3 | Computer Engineering |  |  |

# 1. Table of Contents

1. Table of Contents	2

2. Purpose	3

3. Theory	3

4. Experimental Setup	3

4.1. List Of Materials	4

4.2. Procedure	4

5. Calculations and Results	5

Part 1	5

6. Discussion and Conclusion	8

7. References	9

8. Appendix	10

Experiment Sheet	10

# 2. Purpose

The purpose of this experiment is to investigate the charging and discharging behavior of a capacitor in an RC circuit, and to examine the influence of capacitance, resistance, and voltage on the process. By conducting controlled trials with varying parameters, such as different capacitance values and source voltages, we aim to analyze how these factors affect the time constant of the circuit, the voltage across the capacitor, and the current passing through the circuit. Through experimental observations and data analysis, we seek to validate theoretical models and gain a deeper understanding of the fundamental principles governing capacitor behavior in electrical circuits.

# 3. Theory

A capacitor is an electronic component that stores electrical energy in an electric field. It consists of two conductive plates separated by a dielectric material. When a voltage is applied across the plates, one plate accumulates positive charge (+Q) while the other accumulates negative charge (-Q), creating an electric field between them.

Capacitance (C): Capacitance is a measure of a capacitor's ability to store charge per unit voltage. It is defined as the ratio of the charge stored on the plates (Q) to the voltage across the plates (V):

The unit of capacitance is the Farad (F) in the SI system.

RC Circuit: In an RC circuit, a resistor (R) and a capacitor (C) are connected in series with a voltage source. When the circuit is closed, the capacitor charges or discharges through the resistor depending on the applied voltage.

Charging of Capacitor: During charging, the capacitor initially behaves like a short circuit, allowing current to flow through the circuit. As the capacitor charges, the voltage across it increases, and the current gradually decreases according to an exponential decay function. The rate at which the capacitor charges is determined by the RC time constant (τ), which is the product of the resistance (R) and the capacitance (C):

The time constant represents the time it takes for the capacitor to charge to approximately 63.2% of the applied voltage.

Discharging of Capacitor: During discharging, the capacitor releases the stored charge through the resistor, causing the voltage across it to decrease exponentially over time. The discharge process follows the exponential decay function determined by the RC time constant.

The current passing through the circuit during discharging is given by:

Where:

- I(t) is the current passing through the circuit at time t.

- Vs​ is the source voltage.

- R is the resistance.

- τ is the time constant of the circuit.

The voltage across the capacitor during discharging is given by:

Where:

- Vc​ is the voltage across the capacitor at time t.

- Vs​ is the source voltage.

- τ is the time constant of the circuit.

These formulas describe the behavior of the circuit during the discharging phase, where the capacitor releases its stored energy through the resistor, causing the voltage across the capacitor and the current passing through the circuit to decrease over time.

# 4. Experimental Setup

## 4.1 Equipment List

- Board x1

- DC Voltage Sour(0-30V range) x1

- Digital Multimeter x2

- On-Off switch x1

- 1000μF capacitor x2

- Connection cables

- 10kΩ Resistor x2

## 4.2 Procedure

- 1. Ensure the proper functioning of the capacitor and resistance by checking with a multimeter.

- 2. Precisely adjust the voltage source to the desired value and verify its functionality using a digital multimeter.

- 3. Construct the circuit depicted in Figure 3, ensuring correct orientation of the capacitor. Incorrect orientation may result in circuit malfunction. Refer to Figures 4 and 5 for detailed connections.

- 4. Adhere to the instructions outlined in Parts I to V.

- 5. Record a video for each part of the experiment, capturing the start and end. Ensure visibility of the power supply voltage, multimeter screens, and the circuit, as illustrated in Figure 4.

- 6. Display voltage and current values on the frames corresponding to the time in the tables. Utilize image processing and OCR programs to extract these values from recordings.

- 7. Generate voltage vs. time and current vs. time graphs for each scenario. Analyze how voltage across and current through the capacitor vary with time in all cases.

- 8. Calculate theoretical time constants (τ) for each case. Determine experimental time constants by plotting 𝑙𝑛 (𝐼𝑒𝑥𝑝) vs. time graph and calculating the slope, which equals to − 1/τ according to Equation 5. Calculate percentage error of time constants

# 5. Calculations and Results

## 5.1 Tables

Table 1 R=10kΩ, C=1000 μF

| Time (s) | I(mA) | VC(V) |
| --- | --- | --- |
| 5 | 670 | 3.49 |
| 10 | 380 | 6.3 |
| 15 | 240 | 7.64 |
| 20 | 150 | 8.52 |
| 25 | 90 | 9.09 |
| 30 | 60 | 9.37 |
| 35 | 40 | 9.61 |
| 40 | 30 | 9.79 |
| 45 | 20 | 9.82 |
| 50 | 10 | 9.87 |
| 55 | 10 | 9.91 |
| 60 | 7 | 9.99 |
| 65 | 5 | 9.95 |
| 70 | 3 | 9.96 |
| 75 | 2 | 9.97 |
| 80 | 2 | 9.97 |

Table 2 R=20kΩ, C=1000 μF

| Time (s) | I(mA) | Vc(V) |
| --- | --- | --- |
| 5 | 400 | 2.09 |
| 10 | 310 | 3.97 |
| 15 | 240 | 5.23 |
| 20 | 190 | 6.29 |
| 25 | 140 | 7.11 |
| 30 | 110 | 7.73 |
| 35 | 90 | 8.23 |
| 40 | 70 | 8.61 |
| 45 | 50 | 8.92 |
| 50 | 43 | 9.12 |
| 55 | 32 | 9.33 |
| 60 | 24 | 9.48 |
| 65 | 20 | 9.58 |
| 70 | 15 | 9.66 |
| 75 | 10 | 9.73 |
| 80 | 7 | 9.78 |

Table 3 R=20kΩ, C=2000 μF, charge and discharge

| - Charge |  |  |
| --- | --- | --- |
| Time (s) | I(mV) | Vc(V) |
| 5 | 450 | 1.20 |
| 10 | 390 | 2.27 |
| 15 | 340 | 3.32 |
| 20 | 300 | 4.02 |
| 25 | 260 | 4.83 |
| 30 | 230 | 5.36 |
| 35 | 200 | 5.95 |
| 40 | 180 | 6.43 |
| 45 | 160 | 6.88 |
| 50 | 140 | 7.22 |
| 55 | 120 | 7.57 |
| 60 | 110 | 7.83 |
| 65 | 90 | 8.08 |
| 70 | 80 | 8.32 |
| 75 | 70 | 8.51 |
| 80 | 60 | 8.68 |

| - Discharge |  |  |
| --- | --- | --- |
| Time (s) | I(mV) | Vc(V) |
| 5 | 445 | 8.76 |
| 10 | 393 | 7.73 |
| 15 | 340 | 6.63 |
| 20 | 301 | 5.89 |
| 25 | 266 | 5.24 |
| 30 | 233 | 4.58 |
| 35 | 204 | 4.01 |
| 40 | 179 | 3.54 |
| 45 | 160 | 3.19 |
| 50 | 140 | 2.76 |
| 55 | 126 | 2.46 |
| 60 | 110 | 2.15 |
| 65 | 96 | 1.89 |
| 70 | 86 | 1.68 |
| 75 | 75 | 1.49 |
| 80 | 67 | 1.32 |

Table 4 Charging of capacitor when the voltage dropped to 5V

| Time (s) | I(mA) | Vc(V) |
| --- | --- | --- |
| 5 | 220 | 0.69 |
| 10 | 200 | 1.09 |
| 15 | 170 | 1.59 |
| 20 | 150 | 2.00 |
| 25 | 130 | 2.38 |
| 30 | 110 | 2.70 |
| 35 | 100 | 2.99 |
| 40 | 90 | 3.21 |
| 45 | 80 | 3.44 |
| 50 | 70 | 3.63 |
| 55 | 60 | 3.79 |
| 60 | 50 | 3.93 |
| 65 | 40 | 4.06 |
| 70 | 38 | 4.17 |
| 75 | 35 | 4.28 |
| 80 | 32 | 4.36 |

Table 5 % error of time constants

| Part | Value of Voltage,Resistor and Capacitor |  |  | Time Constant |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Theoretical | Experimental | % error |
| 1 | V1: 10 V | R1: 10kΩ | C1: 1000 μF |  |  |  |
| 2 | V1: 10 V | R2: 20kΩ | C1: 1000 μF |  |  |  |
| 3 A | V1: 10 V | R2: 20kΩ | C2: 2000 μF |  |  |  |
| 3 B | V1: 10V | R2: 20kΩ | C2: 2000 μF |  |  |  |
| 4 | V2: 5V | R2: 20kΩ | C2: 2000 μF |  |  |  |

# 6. Discussion and Conclusion

This experiment explored the charging and discharging behavior of a capacitor in an RC circuit. Key findings include:

- Capacitance, resistance, and voltage significantly affect charging and discharging rates.

- The RC time constant accurately predicts the time it takes for the capacitor to reach approximately 63.2% of its maximum charge.

- Voltage and current follow exponential decay functions during charging and discharging.

- Experimental data closely align with theoretical predictions, validating fundamental principles of capacitor behavior in electrical circuits.

In summary, this experiment deepened our understanding of capacitor dynamics in RC circuits and provided valuable insights into electrical phenomena.

# 7. References

- https://canvas.instructure.com/files/248988405/download?download_frd=1

- https://canvas.instructure.com/files/245848691/download?download_frd=1

# 8. Appendix

## Experiment Sheets