MARMARA UNIVERSITY

FACULTY OF ENGINEERING

COMPUTER ENGINEERING

PHYS1104

PHYSICS LABORATORY

EXPERIMENT 3

Non-Ohmic Device in A Circuit

SXGXE3

| NAME | STUDENT NUMBER |
| --- | --- |

Submitted to: Hüseyin YALTIRIK

1-) PURPOSE

This experiment aims showing similarities and differences between light bulb (which has non-ohmic behavior) and regular resistor in circuit.

2-) CALCULATIONS & RESULTS

Vtotal=Vbulb+Vresistor

Rbulb=Rroom[1+α(T-Troom)]

Troom=293K, Rroom=3.1Ω,α=4.5⋅10-3⋅K-1

Table 1 Voltage and current values of the resistor and bulb

|  | Measured values |  |  |  |  | Bulb |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Vdc | A | Vresistor | Vbulb | Vtotal | Verror | Rbulb | Temp (°C) |
| 1,00 | 0,06 | 0,58 | 0,35 | 0,93 | 7% | 5,83 | 215,70 |
| 2,00 | 0,09 | 0,85 | 0,99 | 1,84 | 8% | 11,00 | 586,31 |
| 3,00 | 0,12 | 1,13 | 1,66 | 2,79 | 7% | 13,83 | 789,18 |
| 4,00 | 0,14 | 1,33 | 2,43 | 3,76 | 6% | 17,36 | 1042,22 |
| 5,00 | 0,16 | 1,57 | 3,08 | 4,65 | 7% | 19,25 | 1177,71 |
| 6,00 | 0,18 | 1,74 | 3,87 | 5,61 | 6% | 21,50 | 1339,00 |
| 7,00 | 0,20 | 1,96 | 4,64 | 6,60 | 6% | 23,20 | 1460,86 |
| 8,00 | 0,22 | 2,11 | 5,42 | 7,53 | 6% | 24,64 | 1564,09 |
| 9,00 | 0,24 | 2,30 | 6,11 | 8,41 | 7% | 25,46 | 1622,87 |
| 10,00 | 0,25 | 2,47 | 7,09 | 9,56 | 4% | 28,36 | 1830,75 |
| 11,00 | 0,27 | 2,56 | 7,80 | 10,36 | 6% | 28,89 | 1868,75 |
| 12,00 | 0,29 | 2,78 | 8,59 | 11,37 | 5% | 29,62 | 1921,08 |
| 13,00 | 0,30 | 2,85 | 9,41 | 12,26 | 6% | 31,37 | 2046,52 |
| 14,00 | 0,32 | 3,03 | 10,10 | 13,13 | 6% | 31,56 | 2060,14 |
| 15,00 | 0,33 | 3,20 | 11,19 | 14,39 | 4% | 33,91 | 2228,60 |
| 16,00 | 0,35 | 3,29 | 11,87 | 15,16 | 5% | 33,91 | 2228,60 |
| 17,00 | 0,36 | 3,37 | 12,50 | 15,87 | 7% | 34,72 | 2286,67 |
| 18,00 | 0,38 | 3,58 | 13,69 | 17,27 | 4% | 36,03 | 2380,57 |
| 19,00 | 0,39 | 3,67 | 14,21 | 17,88 | 6% | 36,44 | 2409,96 |
| 20,00 | 0,39 | 3,83 | 15,01 | 18,84 | 6% | 38,49 | 2556,92 |

3-) CONCLUSION

As we can see when the voltage of the circuit goes up at the same time a current, voltage of resistor, the voltage of bulb and resistor of the bulb goes up. So, voltage and current have inverse proportion for resistor and bulb.

But as we can see on the graphs the resistor has linear proportion, but the bulb has parabolical proportion. This means resistor has a constant ohmic resistor, but a bulb has not. Rbulb is not constant it is increasing while we increase the voltage of the circuit.

4-) REFERENCES

- Marmara University Faculty of Engineering Laboratory Manual

- https://youtu.be/IH-K6ANrI6I